Software Verification and Validation: Open-Ended Lab

Smart Autonomous Cold-Chain Delivery and Control System (SACDCS)

Task 4: Operation Schema for the Conservation Chamber

Abdullah Nadeem Lodhi  |  SP24/BSSE/006  |  Section A

# Chamber State Variables

| Variable | Values |
| --- | --- |
| T1, T2 | Readings of the two temperature sensors (°C) |
| Tavg | Average of the valid readings |
| low, high | Current band limits (2 and 8, or 2 and 6) |
| cooler | OFF, COOLING |
| door | OPEN, CLOSED_UNLOCKED, CLOSED_LOCKED |
| doorSwitch | Door contact sensor: OPEN or CLOSED |
| lockSensor | Lock bolt sensor: LOCKED or UNLOCKED |
| alarm | NONE, WARNING, CRITICAL |
| cargo | EMPTY, OK, COMPROMISED |
| power | MAIN, BACKUP |
| authOK | TRUE or FALSE for the current clinic code |
| excursionTime | Seconds in a row that Tavg has been outside the band |

# The 12 Operations

| Op ID | Operation | Purpose | Allowed robot states |
| --- | --- | --- | --- |
| OP-01 | InitialiseChamber | Run the self-test of sensors, cooler and lock | S1 |
| OP-02 | LoadCargo | Open the door at the warehouse, load, record the manifest | S1 |
| OP-03 | SealChamber | Close and lock the door, check the seal | S1 |
| OP-04 | SetTemperatureBand | Apply the band from the manifest | S1, S2 |
| OP-05 | ReadTemperature | Read both sensors and decide the value of Tavg | S1 to S11 |
| OP-06 | RegulateTemperature | Switch the cooler on or off to hold the band | S1 to S11 |
| OP-07 | RaiseTemperatureAlarm | Raise WARNING or CRITICAL, flag the cargo | S1 to S11 |
| OP-08 | SwitchToBackupPower | Move the chamber to its own battery if drive power drops | S3 to S11 |
| OP-09 | AuthenticateRecipient | Check the clinic code | S7 |
| OP-10 | UnlockChamber | Unlock and open the door for the clinic staff | S7 to S8 |
| OP-11 | CloseAndLockChamber | Close and lock the door after handover | S8 |
| OP-12 | ConfirmDelivery | Write the delivery log and send the confirmation | S8 to S9 |

# Operation Schemas for the Six Key Operations

Notation: ? marks an input, ! marks an output, Δ lists state variables that may change, Ξ lists variables that stay the same. Primed names (cooler') are the values after the operation.

### OP-05 ReadTemperature

| Part | Content |
| --- | --- |
| Inputs | rawT1?, rawT2? (sensor readings) |
| Outputs | Tavg!, sensorStatus! |
| Δ | Tavg, excursionTime |
| Ξ | door, cooler, cargo |
| Precondition | The chamber is powered and the sample timer (5 s) has fired. |
| Postcondition | If ¦rawT1? − rawT2?¦ ≤ 1.0: Tavg' = (rawT1? + rawT2?) / 2 and sensorStatus! = OK. If the difference is more than 1.0: Tavg' = max(rawT1?, rawT2?) and sensorStatus! = DISAGREE. If one sensor gives no valid reading: Tavg' = the other reading and sensorStatus! = SINGLE. If both fail: sensorStatus! = FAULT. In all cases, excursionTime' = excursionTime + 5 when Tavg' is outside [low, high], otherwise 0. |
| Linked states | S1 to S11 |

### OP-06 RegulateTemperature

| Part | Content |
| --- | --- |
| Inputs | Tavg?, low?, high? |
| Outputs | coolerCmd! |
| Δ | cooler |
| Ξ | door, cargo, alarm |
| Precondition | sensorStatus is OK, DISAGREE or SINGLE. |
| Postcondition | If Tavg? ≥ high? − 1.0 then cooler' = COOLING. If Tavg? ≤ low? + 1.0 then cooler' = OFF. Otherwise cooler' = cooler. If sensorStatus is FAULT then cooler' = COOLING and an alarm is requested. |
| Linked states | S1 to S11 (the cooler continues in Link Lost and Safe Mode) |

### OP-07 RaiseTemperatureAlarm

| Part | Content |
| --- | --- |
| Inputs | Tavg?, excursionTime?, low?, high? |
| Outputs | alarmMsg! |
| Δ | alarm, cargo |
| Ξ | door, cooler |
| Precondition | Cargo is OK or COMPROMISED (not EMPTY). |
| Postcondition | If Tavg? is within 0.5 °C of low? or high? and excursionTime? < 30: alarm' = WARNING. If excursionTime? ≥ 30: alarm' = CRITICAL and cargo' = COMPROMISED. The message is logged and sent to Mission Control within 2 s. A CRITICAL alarm cannot be cleared by the robot itself. |
| Linked states | S1 to S11. In S7 the door stays locked after a CRITICAL alarm until Mission Control decides. |

### OP-03 SealChamber

| Part | Content |
| --- | --- |
| Inputs | sealCmd? |
| Outputs | sealStatus! |
| Δ | door |
| Ξ | cargo, cooler |
| Precondition | doorSwitch = CLOSED and the robot is stopped. |
| Postcondition | lockSensor = LOCKED and door' = CLOSED_LOCKED. If lockSensor does not show LOCKED within 3 s, sealStatus! = FAILED and the robot cannot leave S1. |
| Linked states | S1 only |

### OP-10 UnlockChamber

| Part | Content |
| --- | --- |
| Inputs | unlockCmd? |
| Outputs | unlockStatus! |
| Δ | door |
| Ξ | cargo, alarm |
| Precondition | state = S7, authOK = TRUE, robot is inside the clinic area, speed = 0. |
| Postcondition | lockSensor = UNLOCKED and door' = CLOSED_UNLOCKED, then OPEN when doorSwitch = OPEN. unlockStatus! = UNLOCKED. If any precondition is false, door' = door and the command is rejected and logged. If cargo = COMPROMISED, the unlock needs an override from Mission Control. |
| Linked states | S7 to S8 |

### OP-11 CloseAndLockChamber

| Part | Content |
| --- | --- |
| Inputs | closeCmd? |
| Outputs | lockStatus! |
| Δ | door |
| Ξ | cooler, alarm |
| Precondition | state = S8 and the staff have confirmed the cargo was removed. |
| Postcondition | doorSwitch = CLOSED and lockSensor = LOCKED, then door' = CLOSED_LOCKED and lockStatus! = LOCKED. If either sensor disagrees after 5 s, lockStatus! = FAILED and an alert is sent. The robot may not leave S8 until lockStatus! = LOCKED. |
| Linked states | S8 to S9 |

# Sensor Verification

The system never takes a command as proof that it worked. Every postcondition is checked against a sensor.

| Operation | What is checked | Sensors | If the check fails |
| --- | --- | --- | --- |
| OP-05 | The two readings must agree within 1.0 °C. A reading that is stuck or outside −10 to 40 °C is invalid. | T1, T2 | One valid sensor: use it and raise a WARNING. Disagree: use the higher value for alarm decisions. Both invalid: CRITICAL alarm and S11. |
| OP-06 | After the cooler switches on, Tavg should fall when it is above the band. Switching on is checked by the cooler current and by the temperature trend over the next minute. | Cooler current sensor, T1, T2 | If the temperature does not fall, raise a WARNING, and a CRITICAL alarm when the 30 s limit is passed. |
| OP-07 | The alarm is accepted as sent when Mission Control acknowledges it. If there is no acknowledgement in 10 s, the alarm is repeated and stored locally. | Link status | Keep repeating, and Link Lost handles a dead link. |
| OP-03 | A locked door needs two signals: the door contact shows CLOSED and the lock bolt sensor shows LOCKED. One signal alone is not enough. | doorSwitch, lockSensor | sealStatus! = FAILED and the robot cannot depart. |
| OP-10 | The door counts as unlocked only when the lock bolt sensor shows UNLOCKED. It counts as open when the door contact shows OPEN. | lockSensor, doorSwitch | Command is logged as failed, a maintenance alert is sent. |
| OP-11 | Same two-signal rule as OP-03, plus the staff confirmation. A door that is closed but not locked still counts as not locked. | doorSwitch, lockSensor | Robot stays in S8 and alerts Mission Control. |