Software Verification and Validation: Open-Ended Lab

Smart Autonomous Cold-Chain Delivery and Control System (SACDCS)

Task 1: Requirement Extraction

Abdullah Nadeem Lodhi  |  SP24/BSSE/006  |  Section A

# Scenario

A national health authority uses a fleet of autonomous delivery robots to carry vaccines and temperature-sensitive medicines from a central warehouse to 20 remote clinics. Each robot carries a sealed conservation chamber. The robots are watched remotely from Mission Control over a link with a delay of several seconds. Some routes cross unmanned railway level crossings, which are managed by an external system called ARLCCS.

# Assumptions

The brief gave no numbers for some values, so these were chosen and are used the same way in all five files.

| Item | Value |
| --- | --- |
| Normal cargo temperature band | 2.0 °C to 8.0 °C |
| Chamber temperature sensors | 2 (T1 and T2), average used as Tavg |
| Temperature sampling and telemetry period | every 5 s |
| Maximum speed | 15 km/h (about 4.17 m/s) |
| Mission Control link delay | up to 5 s one way |
| Heartbeat from Mission Control | every 3 s |
| Link Lost timeout (version 1.0) | 30 s |
| Level crossing hold distance | 10 m before the stop line |

# Functional Requirements

| Req. ID | Requirement | Type | Priority | Verification Method |
| --- | --- | --- | --- | --- |
| FR-01 | The robot shall accept a mission from Mission Control only if the mission contains a route, a destination clinic, a cargo manifest and the required temperature band. A mission with any of these missing shall be rejected and the reason reported. | Functional | High | Test |
| FR-02 | Before leaving the warehouse the robot shall run a self-check of both temperature sensors, the door lock, the brakes, the obstacle sensors and the battery (at least 80%). The robot shall not depart if any check fails. | Functional | Critical | Test |
| FR-03 | The robot shall drive along the assigned route to the destination clinic without human control and shall keep its speed at or below 15 km/h. | Functional | High | Simulation and field test |
| FR-04 | The robot shall detect obstacles in its path, slow down, and stop if the obstacle does not move. It shall report a blocked route to Mission Control after 60 s. | Functional | High | Test |
| FR-05 | The system shall keep the chamber temperature between 2 °C and 8 °C for the whole mission and shall sample both sensors every 5 s. | Functional | Critical | Test |
| FR-06 | The system shall raise a WARNING when Tavg comes within 0.5 °C of either limit and a CRITICAL alarm when Tavg stays outside the band for 30 s in a row. Each alarm shall be logged and sent to Mission Control within 2 s of being raised. | Functional | Critical | Test |
| FR-07 | The robot shall send telemetry (position, speed, state, battery, T1, T2, door status) to Mission Control every 5 s. | Functional | Medium | Test |
| FR-08 | At every level crossing the robot shall stop at the hold point, send a request to ARLCCS and wait for a reply before moving. | Functional | Critical | Test and simulation |
| FR-09 | The chamber door shall unlock at the clinic only after the clinic code is checked. The robot shall then record and send a delivery confirmation (time, clinic, temperature summary). | Functional | High | Demonstration and test |
| FR-10 | The robot shall accept the remote commands Pause, Resume, Reroute and Abort from Mission Control, and shall refuse any command that is unsafe in its current state. | Functional | Medium | Test |

# Non-Functional Requirements

| Req. ID | Requirement | Type | Priority | Verification Method |
| --- | --- | --- | --- | --- |
| NFR-01 | The time from the obstacle sensor detecting an object to the brake command being issued shall be 100 ms or less. | Non-functional (performance) | Critical | Test and timing analysis |
| NFR-02 | If drive power is lost, the chamber shall stay inside its temperature band for at least 8 hours on its own backup power. | Non-functional (reliability) | High | Test and analysis |
| NFR-03 | All commands and telemetry between the robot and Mission Control shall be encrypted and authenticated. A message that fails authentication shall be dropped and logged. | Non-functional (security) | High | Inspection and test |
| NFR-04 | Mission Control shall show any alarm within 3 s of receiving it and shall always display the current link delay to the operator. | Non-functional (usability) | Medium | Demonstration |
| NFR-05 | Logs of temperature, state changes, alarms and deliveries shall be tamper-evident and kept for 24 months. | Non-functional (auditability) | Medium | Inspection and analysis |

# High-Risk and Safety Requirements

| Req. ID | Requirement | Type | Priority | Verification Method |
| --- | --- | --- | --- | --- |
| HR-01 | The robot shall not enter a level crossing unless ARLCCS has granted permission and the crossing is reported clear. | Safety | Critical | Test and simulation |
| HR-02 | If the chamber stays outside its temperature band for 30 s, the system shall mark the cargo as COMPROMISED and block normal delivery until Mission Control decides what to do. | Safety (cargo) | Critical | Test |
| HR-03 | If no heartbeat from Mission Control is received for 30 s, the robot shall enter Link Lost, stop at the nearest safe stopping point (never on a crossing or within 30 m of one) and keep the chamber cooling. | Safety | Critical | Test and simulation |
| HR-04 | The robot shall detect a person or object within 8 m of its path and stop at least 1 m before it (stopping distance of 3 m or less at 15 km/h), without waiting for any command from Mission Control. | Safety | Critical | Test |