Software Verification and Validation: Open-Ended Lab

Smart Autonomous Cold-Chain Delivery and Control System (SACDCS)

Task 3: State Modelling and Transition Verification

Abdullah Nadeem Lodhi  |  SP24/BSSE/006  |  Section A

# Robot States

S0 to S9 form the base model. S10 and S11 are the extensions for Link Lost and Safe Mode.

| State ID | State | Meaning | Entry condition | Exit condition |
| --- | --- | --- | --- | --- |
| S0 | Idle | Robot is docked at the warehouse with no mission. | Power on, delivery finished, or mission cancelled. | A mission is received. |
| S1 | Loading and Pre-Check | Cargo is loaded and the self-check runs. | Mission accepted in S0. | Self-check passes or fails. |
| S2 | Ready | Everything checked, robot waits for permission to leave. | Self-check passed. | Departure authorised, or mission aborted. |
| S3 | En Route | Robot drives to the clinic. | Departure authorised, or crossing finished, or obstacle cleared (on the outbound trip). | Crossing reached, obstacle found, arrival at the clinic, or a fault. |
| S4 | Crossing Hold | Robot is stopped at the hold point and waits for ARLCCS. | Crossing hold point reached. | A fresh CLEAR is received. |
| S5 | Crossing | Robot is driving over the crossing. | Fresh CLEAR received in S4. | Robot is fully across. |
| S6 | Obstacle Stop | Robot is stopped because an obstacle or person is in the way. | Obstacle or person detected in the safety zone. | Path clear for 5 s. |
| S7 | At Clinic | Robot is at the clinic with the door locked, waiting for the code. | Arrival at the clinic. | Correct code entered. |
| S8 | Unloading | Door is open and the clinic staff take the cargo. | Correct code in S7. | Cargo removed and the door closed and locked. |
| S9 | Returning | Robot drives back to the warehouse. | Door closed and locked in S8. | Arrival at the warehouse, or the same events as S3. |
| S10 | Link Lost | Robot has lost contact with Mission Control and has stopped. | No heartbeat for 15 s. | Link restored and the operator gives Resume, or 10 minutes pass. |
| S11 | Safe Mode | Brakes on, door locked, cooling on, beacon flashing. | Critical fault, or Link Lost for 10 minutes, or battery under 10%. | Authorised reset by an operator. |

# State and Event

A state is a condition the robot stays in for some time, for example En Route or Link Lost. An event is something that happens at one moment and may change the state, for example "heartbeat timeout" or "obstacle detected". A state can last for minutes. An event takes no time. The robot can sit in a state without anything happening, but a state change always needs an event.

| Event ID | Event |
| --- | --- |
| E01 | Mission received |
| E02 | Self-check passed |
| E03 | Self-check failed |
| E04 | Departure authorised |
| E05 | Crossing hold point reached |
| E06 | Fresh CLEAR from ARLCCS |
| E07 | Permission denied or expired |
| E08 | Crossing cleared |
| E09 | Obstacle or person detected |
| E10 | Path clear for 5 s |
| E11 | Arrived at the clinic |
| E12 | Correct clinic code entered |
| E13 | Wrong clinic code |
| E14 | Cargo removed, door closed and locked |
| E15 | Arrived at the warehouse |
| E16 | Heartbeat timeout (15 s) |
| E17 | Link restored |
| E18 | Critical fault (both temperature sensors failed, door open while moving, brake fault) |
| E19 | Battery under 10% |
| E20 | Authorised operator reset |
| E21 | Abort command |
| E22 | Operator Resume command |

# State Transition Table

P means the driving state stored before the robot left it (S3 or S9).

| Current state | Event | Guard | Next state | Action |
| --- | --- | --- | --- | --- |
| S0 | E01 | Mission is complete | S1 | Store manifest, start self-check |
| S1 | E02 | None | S2 | Report ready |
| S1 | E03 | None | S0 | Report fault, block mission |
| S2 | E04 | Mission Control approval received | S3 | Start navigation and telemetry |
| S2 | E21 | None | S0 | Cancel mission |
| S3 | E05 | None | S4 | Stop at hold point, send request to ARLCCS |
| S3 | E09 | None | S6 | Brake, store P = S3 |
| S3 | E11 | None | S7 | Stop, keep door locked |
| S4 | E06 | CLEAR is 20 s old or less | S5 | Start crossing at crawl speed |
| S4 | E07 | None | S4 | Ask again every 10 s, report to Mission Control after 5 minutes |
| S5 | E08 | None | P | Return to normal speed |
| S6 | E10 | None | P | Resume driving |
| S7 | E12 | Robot is inside the clinic area | S8 | Unlock door, log it |
| S7 | E13 | None | S7 | Keep door locked, alert after 3 tries |
| S8 | E14 | Door sensor and lock sensor both confirm | S9 | Send delivery confirmation |
| S9 | E05 | None | S4 | Same as S3, store P = S9 |
| S9 | E09 | None | S6 | Same as S3, store P = S9 |
| S9 | E15 | None | S0 | Dock, send mission report |
| S3, S4, S6, S7, S9 | E16 | None | S10 | Stop at the nearest safe point, keep cooling |
| S5 | E16 | None | S5 | Finish crossing first, then enter S10 |
| S8 | E16 | None | S8 | Wait for the door to close, then S10 |
| S10 | E17 | None | S10 | Report status, wait for Resume |
| S10 | E22 | Link is up | P | Resume |
| S10 | 10 minutes pass | None | S11 | Enter Safe Mode |
| S1 to S10 | E18 | None | S11 | Brake, lock door, alert, flash beacon |
| S3, S4, S6, S7, S9, S10 | E19 | None | S11 | Disable drive, keep cooling |
| S11 | E20 | Robot at the warehouse | S0 | Mission must be issued again |
| S11 | E20 | Robot away from the warehouse | S9 | Return to the warehouse, mission not resumed |

# Safety Questions from the Verifier

The exact verifier questions were not in the brief text, so these are the usual safety checks for this model. Replace them if your lab sheet lists different ones.

### 1. Can the robot reach S5 without going through S4?

No. The only transition into S5 comes from S4 with a fresh CLEAR. This is the core of HR-01.

### 2. Can the robot ever stop on the crossing?

A Link Lost during S5 does not stop the robot, because the state table keeps it in S5 until it is across. If the robot is blocked on the crossing, it stays in S5 and sends an emergency alert instead of entering Link Lost. This is the one place where stopping is allowed, and TC-05 tests it.

### 3. Can the robot move with the door open?

No. The door opens only in S1 and S8, and the robot is stopped in both. Leaving S8 needs the door sensor and the lock sensor to agree. A door-open signal while moving is event E18 and sends the robot to S11.

### 4. Can the door open in S10 or S11?

No. Door unlock is only reached from S7 with E12. S10 and S11 keep the door locked.

### 5. What happens if Link Lost happens near a crossing?

The robot stops at the nearest safe stopping point, which is never on the crossing and never within 30 m of it. If it is already at the hold point in S4, it simply stays there.

### 6. Are there deadlocks?

S4 can wait forever if ARLCCS never answers, so a report goes to Mission Control after 5 minutes and Link Lost still applies. S6 waits for the path to clear, with a report after 60 s. S11 can only be left by an operator reset, which is on purpose.

### 7. What if the temperature goes out of range in S7?

The state does not change. The cargo is marked COMPROMISED, a CRITICAL alarm is sent, and the door stays locked until Mission Control decides.

### 8. Can the robot leave the warehouse without a passed self-check?

No. S2 is reached only from S1 with E02, and S3 only from S2.